Line Follower Robot Report – Perfect LFR with Fixed 90° Turns

1. Performance Requirements

| Requirement ID | Parameter | Target Value / Behavior | Notes |
| --- | --- | --- | --- |
| PR-01 | Maximum speed | 210 (PWM) | FAST_SPEED, straight line |
| PR-02 | Normal speed | 180 (PWM) | Smooth tracking |
| PR-03 | Minimum speed | 140 (PWM) | For tight curves |
| PR-04 | Turn precision | ±5° | Fixed 90° turns |
| PR-05 | Line detection | 5 sensors active | Detect intersections accurately |
| PR-06 | Obstacle detection | ≤7 cm | Ultrasonic sensor |
| PR-07 | Sharp curve handling | Edge sensor detection | Recover line quickly |
| PR-08 | Intersection handling | ≥4 sensors active | Correct turn executed |

2. Safety Requirements

| Requirement ID | Safety Feature | Implementation | Notes |
| --- | --- | --- | --- |
| SR-01 | Emergency stop | stopMotors() | Stops both motors instantly |
| SR-02 | Obstacle detection | Ultrasonic sensor | Avoids collisions |
| SR-03 | Line lost recovery | Spin in last turn direction | Prevents robot from leaving track |
| SR-04 | Sensor fault detection | Sensor read check | Avoids false turn detection |
| SR-05 | Motor protection | constrain() | Ensures PWM within 0–255 |

3. PID Tuning Process

| Step | Parameter | Initial Value | Adjusted Value | Notes |
| --- | --- | --- | --- | --- |
| 1 | Kp (Proportional) | 30 | 40 | Stronger correction for deviation |
| 2 | Kd (Derivative) | 10 | 15 | Reduces overshoot on curves |
| 3 | Ki (Integral) | 0 | 0 | Not used |
| 4 | Testing | Straight line | Observed line following | Iterative tuning |
| 5 | Fine-tuning | Intersections | Adjust Kp/Kd | Smooth turns and minimal oscillation |

4. Functional Test Results (Sample Data)

| Test ID | Function Tested | Input Condition | Expected Behavior | Actual Observed Behavior | Result |
| --- | --- | --- | --- | --- | --- |
| FT-01 | Line following | Straight track | Robot follows line | Follows line with small oscillations | Pass |
| FT-02 | Sharp left curve | Only rightmost sensor active | Spins left until line found | Robot recovers line within 0.3 sec | Pass |
| FT-03 | Sharp right curve | Only leftmost sensor active | Spins right until line found | Robot recovers line within 0.35 sec | Pass |
| FT-04 | 90° left turn | Left sensors active | Executes precise left turn | Turn completed in 0.55 sec, centered on line | Pass |
| FT-05 | 90° right turn | Right sensors active | Executes precise right turn | Turn completed in 0.58 sec, centered on line | Pass |
| FT-06 | Intersection | ≥4 sensors active | Detect and take correct path | Robot chose correct turn every trial | Pass |
| FT-07 | Line lost | No sensor active | Robot recovers using last direction | Recovery achieved in 0.25 sec | Pass |
| FT-08 | Obstacle avoidance | Obstacle ≤7 cm | Robot avoids obstacle | Avoided left/right based on free path | Pass |

5. Performance Test Results (Sample Data)

| Test ID | Parameter | Test Condition | Measured Value | Pass/Fail | Notes |
| --- | --- | --- | --- | --- | --- |
| PT-01 | Maximum speed | Straight line | 210 PWM | Pass | Reached FAST_SPEED |
| PT-02 | Turn precision | 90° turn | ±3° | Pass | Within ±5° tolerance |
| PT-03 | Intersection handling | 4–5 sensors | 100% success | Pass | No misread |
| PT-04 | Sharp curve recovery | Edge sensor only | 95% success | Pass | Slight delay at extreme angles |
| PT-05 | Obstacle avoidance | Distance ≤7 cm | Avoided obstacle successfully | Pass | Both left & right avoidance tested |
| PT-06 | Line following oscillation | Straight track | ±1–2 units (error) | Pass | Smooth PID response |
| PT-07 | Recovery time | Line lost | 0.25–0.35 sec | Pass | Quic line regain |

6. Troubleshooting Guide

| Problem | Possible Cause | Solution |
| --- | --- | --- |
| Robot not following line | PID values incorrect | Adjust Kp/Kd |
| Robot overshoots turn | High Kp | Reduce Kp or increase Kd |
| Line lost frequently | Sensor misalignment or dirty sensors | Clean and realign sensors |
| Motors not moving | Motor wiring error | Check IN1–IN4 & ENA/ENB connections |
| Obstacle avoidance fails | Ultrasonic misread | Check TRIG/ECHO wiring, servo movement |
| Robot stops at intersection | Sensors misread | Clean sensors, check alignment, test with 4–5 sensor activation |

7. Risk Register

| Risk ID | Risk Description | Probability | Impact | Mitigation |
| --- | --- | --- | --- | --- |
| R-01 | Sensor misalignment | Medium | High | Proper mounting, regular calibration |
| R-02 | PID tuning errors | High | Medium | Iterative testing, fine-tune Kp/Kd |
| R-03 | Motor failure | Low | High | Use quality motors, check wiring |
| R-04 | Obstacle collision | Medium | Medium | Ultrasonic sensor & obstacle routine |
| R-05 | Line detection failure | Low | High | Keep track clean, recalibrate sensors regularly |
| R-06 | Servo failure | Low | Medium | Test servo movement, ensure correct PWM |

8. Project Timeline

| Task | Start Date | End Date | Duration | Status |
| --- | --- | --- | --- | --- |
| Hardware setup | 01-Jan | 03-Jan | 3 days | Completed |
| Sensor calibration | 04-Jan | 05-Jan | 2 days | Completed |
| PID tuning | 06-Jan | 07-Jan | 2 days | Completed |
| Obstacle avoidance implementation | 08-Jan | 09-Jan | 2 days | Completed |
| Intersection handling | 10-Jan | 11-Jan | 2 days | Completed |
| Testing & debugging | 12-Jan | 13-Jan | 2 days | Completed |
| Final demonstration | 14-Jan | 14-Jan | 1 day | Scheduled |