Heaven’s Light is Our Guide

Rajshahi University of Engineering & Technology

Department of

Electrical & Computer Engineering

Project Title

Course Code: ECE 2200

Course Title: Electronic Shop Practice

Submitted to: Submitted by: Md. Omaer Faruq Goni Your Name Assistant Professor Roll: Your Roll Dept of ECE, RUET Semester: 2nd Year EVEN

Submission Date: January 10, 2026

Contents

LIST OF FIGURES vi LIST OF TABLES vi List of Abbreviations and Symbols viii ABBREVIATIONS viii ABSTRACT ix

1 Introduction 1 1.1 Background . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1 1.2 Problem Statement . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1 1.3 Objectives . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2 1.4 Scope and Limitations . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 3 1.5 Organization of Report . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 3

2 Literature Review and Theoretical Background 4 2.1 Fundamental Concepts . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 4 2.1.1 Line Following Principle . . . . . . . . . . . . . . . . . . . . . . . . . 4 2.1.2 IR Sensors (K1 - Natural Sciences) . . . . . . . . . . . . . . . . . . . 4 2.1.3 DC Motors and Motor Control (K3 - Engineering Fundamentals) . . . 4 2.1.4 PWM (K4 - Specialist Knowledge) . . . . . . . . . . . . . . . . . . . . 4 2.1.5 Microcontroller Basics (K4) . . . . . . . . . . . . . . . . . . . . . . . 5 2.1.6 PID Control Theory (K2, K3) . . . . . . . . . . . . . . . . . . . . . . 5 2.1.7 Power Supply and Battery (K1, K3) . . . . . . . . . . . . . . . . . . . 5 2.2 Review of Existing Designs . . . . . . . . . . . . . . . . . . . . . . . . . . . . 6 2.3 PCB Design Fundamentals . . . . . . . . . . . . . . . . . . . . . . . . . . . . 6 2.4 Gap Analysis . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 7

3 System Design and Methodology 8 3.1 Design Approach . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 8 3.2 Requirements Analysis . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 8

3.2.1 Functional Requirements . . . . . . . . . . . . . . . . . . . . . . . . . 8 3.2.2 Performance Requirements . . . . . . . . . . . . . . . . . . . . . . . . 8 3.2.3 Safety Requirements (K7) . . . . . . . . . . . . . . . . . . . . . . . . 9 3.2.4 Cost Constraints (PO11) . . . . . . . . . . . . . . . . . . . . . . . . . 9 3.2.5 Requirements Traceability Matrix . . . . . . . . . . . . . . . . . . . . 9 3.3 System Architecture and Detailed Design . . . . . . . . . . . . . . . . . . . . 9

ii

3.3.1 Overall System Architecture . . . . . . . . . . . . . . . . . . . . . . . 9 3.3.2 Sensor Subsystem Design . . . . . . . . . . . . . . . . . . . . . . . . 10 3.3.3 Microcontroller Subsystem . . . . . . . . . . . . . . . . . . . . . . . . 10 3.3.4 Motor Driver Subsystem . . . . . . . . . . . . . . . . . . . . . . . . . 10 3.3.5 Motor and Wheel Selection . . . . . . . . . . . . . . . . . . . . . . . 10 3.3.6 Power Supply Subsystem . . . . . . . . . . . . . . . . . . . . . . . . . 11 3.3.7 PCB Design . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 11

3.4 PID Control Algorithm Design . . . . . . . . . . . . . . . . . . . . . . . . . . 11 3.4.1 Control System Overview . . . . . . . . . . . . . . . . . . . . . . . . 11 3.4.2 Error Calculation . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 11 3.4.3 Proportional Control . . . . . . . . . . . . . . . . . . . . . . . . . . . 11 3.4.4 Integral Control . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 11 3.4.5 Derivative Control . . . . . . . . . . . . . . . . . . . . . . . . . . . . 12 3.4.6 Complete PID and Motor Speed . . . . . . . . . . . . . . . . . . . . . 12 3.4.7 PID Tuning Strategy . . . . . . . . . . . . . . . . . . . . . . . . . . . 12 3.4.8 Algorithm Flowchart . . . . . . . . . . . . . . . . . . . . . . . . . . . 12

3.5 Design Calculations and Analysis . . . . . . . . . . . . . . . . . . . . . . . . 12 3.5.1 Motor Torque Requirements . . . . . . . . . . . . . . . . . . . . . . . 12 3.5.2 Power Consumption Analysis . . . . . . . . . . . . . . . . . . . . . . 13 3.5.3 Voltage Regulator Heat Dissipation . . . . . . . . . . . . . . . . . . . 13 3.5.4 Turn Radius Calculation . . . . . . . . . . . . . . . . . . . . . . . . . 13 3.5.5 Cost Analysis . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 13

3.6 Alternative Designs Considered . . . . . . . . . . . . . . . . . . . . . . . . . 13 3.7 Risk Assessment . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 14

4 Implementation 15 4.1 Resource Planning and Management . . . . . . . . . . . . . . . . . . . . . . . 15 4.1.1 Component Procurement . . . . . . . . . . . . . . . . . . . . . . . . . 15 4.1.2 Schedule and Timeline Management . . . . . . . . . . . . . . . . . . . 15 4.2 Breadboard Prototyping . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 15 4.3 PCB Assembly . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 16 4.3.1 PCB Fabrication . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 16 4.3.2 Component Soldering . . . . . . . . . . . . . . . . . . . . . . . . . . 16 4.4 Mechanical Assembly . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 16 4.5 Software Development . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 16 4.5.1 Development Environment . . . . . . . . . . . . . . . . . . . . . . . . 16 4.5.2 Code Structure . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 16 4.5.3 Debugging Process . . . . . . . . . . . . . . . . . . . . . . . . . . . . 17 4.6 System Integration . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 17

iii

5 Testing and Results 18 5.1 Testing Methodology . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18 5.2 PID Tuning Results . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18 5.3 Functional Testing . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 19 5.4 Performance Testing . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 19 5.5 Safety and Reliability Testing . . . . . . . . . . . . . . . . . . . . . . . . . . . 19 5.6 Preventive Maintenance Procedures . . . . . . . . . . . . . . . . . . . . . . . 19 5.7 Analysis and Discussion . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 20

6 Project Management and Teamwork 21 6.1 Project Planning . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21 6.2 Budget Management . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21 6.3 Team Organization and Roles . . . . . . . . . . . . . . . . . . . . . . . . . . . 21 6.4 Team Collaboration and Leadership . . . . . . . . . . . . . . . . . . . . . . . 21 6.5 Project Execution and Control . . . . . . . . . . . . . . . . . . . . . . . . . . 22 6.6 Lessons Learned . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 22

7 Economic and Sustainability Analysis 23 7.1 Cost Analysis . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 23 7.1.1 Development Costs . . . . . . . . . . . . . . . . . . . . . . . . . . . . 23 7.1.2 Operational Costs . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 23 7.1.3 Life Cycle Cost . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 23 7.2 Economic Viability . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 23 7.3 Sustainability Assessment . . . . . . . . . . . . . . . . . . . . . . . . . . . . 23 7.3.1 Environmental Impact Assessment . . . . . . . . . . . . . . . . . . . . 23 7.3.2 Social Impact . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 24 7.4 Societal and Environmental Impact Checklist . . . . . . . . . . . . . . . . . . 24 7.5 Ethical Considerations . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 26

8 Conclusions and Recommendations 27 8.1 Summary of Work . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 27 8.2 Achievement of Objectives . . . . . . . . . . . . . . . . . . . . . . . . . . . . 27 8.3 Contributions and Significance . . . . . . . . . . . . . . . . . . . . . . . . . . 27 8.4 Limitations . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 27 8.5 Future Work and Recommendations . . . . . . . . . . . . . . . . . . . . . . . 28 8.6 Personal Reflection and Learning . . . . . . . . . . . . . . . . . . . . . . . . . 28

A Complete Circuit Schematics 29 B PCB Layout Files 30 C Complete Source Code 31

iv

D Component Datasheets 32 E Test Data 33 F Project Photographs 34 G Budget Details 35 H Project Timeline 36 I Risk Register 37 J Meeting Minutes 38 K Video Demonstration Links 39 Assessment Evidence Summary 40 References 42

v

List of Figures

3.1 System Block Diagram . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 10 A.1 Complete System Schematic . . . . . . . . . . . . . . . . . . . . . . . . . . . 29

vi

List of Tables

3.1 Functional Requirements . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 8 3.2 Budget Breakdown . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 9 3.3 Risk Register . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 14

5.1 PID Tuning Process . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18 5.2 Functional Test Results . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 19 5.3 Troubleshooting Guide . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 20

B.1 Bill of Materials (BOM) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 30 vii

List of Abbreviations and Symbols

Abbreviation Full Form

ADC Analog to Digital Converter DC Direct Current

GPIO General Purpose Input/Output H-Bridge A circuit topology for motor control IC Integrated Circuit

IDE Integrated Development Environment IR Infrared

LED Light Emitting Diode

Li-ion Lithium-ion

PCB Printed Circuit Board

PID Proportional-Integral-Derivative PWM Pulse Width Modulation

RPM Revolutions Per Minute

viii

ABSTRACT

Write a concise abstract (not more than 200 words)

Keywords: Artificial Intelligence (AI), Diagnosis of Malaria, Double Hidden Layer Extreme Learning Machine (DELM), Explainaility, Lightweight Convolutional Neural Network, SHAP

ix

Chapter 1

Introduction

1.1 Background

What students write:

• Definition of line follower robots with simple diagrams

• Real-world applications (warehouses, manufacturing, delivery systems) • Brief history of mobile robotics

• Why control systems matter (on-off vs. PID comparison)

• Basic electronics concepts: IR sensors, DC motors, H-bridge, microcontrollers, PWM

Assessment Link:

PO1 - Demonstrates knowledge of electronics fundamentals (K1, K3, K4)

Beginner Tip:

Use 2-3 simple block diagrams. Cite 3-4 basic textbooks or articles.

Page Length: 2-3 pages

1.2 Problem Statement

What students write:

• Clear problem: “Simple on-off line followers have jerky movement, slow speeds, can’t handle curves smoothly”

• Complex Problem Characteristics (explain each):

– P1 (Depth): Requires control theory, sensor physics, motor dynamics, embedded programming

– P2 (Conflicting requirements): Speed vs. accuracy, battery life vs. performance, cost vs. quality

– P3 (Abstract thinking): PID tuning requires understanding mathematical relationships 1

– P4 (Infrequent issues): Sharp turns, intersections, gaps in line

– P6 (Stakeholders): Educational tool (understandable), competition robot (fast), industrial demo (reliable)

• Why simple on-off control is inadequate

• Need for smooth, accurate PID-based solution

Assessment Link:

PO1, PO3 - Identifies complex engineering problem with P1-P7 characteristics

Beginner Tip:

Draw comparison diagram: “Simple Control” vs. “PID Control” behavior Page Length: 1-2 pages

1.3 Objectives

What students write:

• Main Objective: Design and implement PID-controlled line follower with smooth motion and high accuracy

• Specific Objectives (5-7 measurable goals):

1. Design hardware (sensor array, motor driver, power system)

2. Develop PID control firmware

3. Design and fabricate custom PCB

4. Tune PID parameters systematically

5. Test on various track configurations

6. Achieve ≥0.3 m/s speed with <5 cm deviation

7. Complete within budget and timeline

Assessment Link:

PO3, PO11 - Sets clear design and management goals

Beginner Tip:

Make objectives SMART (Specific, Measurable, Achievable, Relevant, Time-bound) 2

1.4 Scope and Limitations

What students write:

• Scope: Indoor use, flat surfaces, 2-3 cm black line on white, 0.2-0.5 m/s range, curves >15 cm radius, rechargeable battery

• Limitations: Not for outdoor/uneven terrain, no obstacle detection, single line only, line width 2-3 cm, no wireless, budget constraint, 12-week timeline

Assessment Link:

Shows realistic understanding of project boundaries

1.5 Organization of Report

What students write:

Brief roadmap of what each chapter contains

3

Chapter 2

Literature Review and Theoretical Background

2.1 Fundamental Concepts

2.1.1 Line Following Principle

What students write:

• How contrast detection works (black absorbs, white reflects) • Basic algorithm: detect position → calculate error → adjust motors

2.1.2 IR Sensors (K1 - Natural Sciences)

What students write:

• Working principle: IR LED + photodiode/phototransistor • Circuit diagram, typical specifications

• Why black/white surfaces behave differently with IR

2.1.3 DC Motors and Motor Control (K3 - Engineering Fundamentals)

What students write:

• DC motor working principle

• Motor specs: voltage, current, RPM, torque

• H-Bridge circuit explanation with diagram

• Why motors need drivers

2.1.4 PWM (K4 - Specialist Knowledge)

What students write:

• What is PWM (diagram showing duty cycle)

4

• How PWM controls speed (average voltage)

• Formula:

�������� = Duty Cycle×������ �� ������ (2.1)

2.1.5 Microcontroller Basics (K4)

What students write:

• Role of microcontroller

• Key peripherals: ADC, GPIO, Timers for PWM

• Brief architecture overview

2.1.6 PID Control Theory (K2, K3)

What students write:

• What is control (maintaining desired state)

• Error definition:

Error = Setpoint−Current Position (2.2)

• P (Proportional): Formula, effect, analogy, limitation

• I (Integral): Formula, effect, eliminates steady-state error

• D (Derivative): Formula, effect, dampens oscillation

• Complete PID formula:

��(��)���� +��������(��)

����(2.3)

• Block diagram

��(��) = ������(��) +����

∫ �� 0

• Basic tuning methods overview

2.1.7 Power Supply and Battery (K1, K3)

What students write:

• Battery types comparison

• Voltage regulation basics

• Power calculation concepts

5

Assessment Link:

PO1 - Applies mathematics, natural science, and engineering fundamentals (K1-K4)

Beginner Tip:

Use lots of diagrams. Explain in your own words after reading references. Page Length: 4-5 pages with diagrams

2.2 Review of Existing Designs

What students write:

• Review 5-6 existing line follower designs (papers, projects, commercial products) • For each design: approach, strengths, weaknesses, what you learned • Comparison table (control type, sensors, speed, accuracy, cost, complexity) • Identify gaps in existing solutions

Assessment Link:

PO1 - Shows research-based knowledge (K8), identifies gaps (P4)

Beginner Tip:

For YouTube projects without papers, describe what you observed Page Length: 3-4 pages

2.3 PCB Design Fundamentals

What students write:

• Why PCB vs. breadboard

• Basic PCB design rules (component placement, trace width, clearance, ground plane) • Design software workflow

• Safety considerations (K7)

Assessment Link:

PO1, PO3 - Demonstrates specialized knowledge (K4, K5, K6)

Page Length: 2 pages

6

2.4 Gap Analysis

What students write:

• Summary of gaps in existing solutions

• How your project addresses these gaps

• Novel aspects of your design (even small improvements count)

Assessment Link:

PO3 - Justifies design approach

Page Length: 1 page

7

Chapter 3

System Design and Methodology

3.1 Design Approach

Instruction

Show: (1) Iterative design process flowchart, (2) Tools used (Arduino IDE, EasyEDA, Proteus), (3) Why these tools selected

Assessment Link:

PO1, PO3 - Systematic problem-solving

3.2 Requirements Analysis

3.2.1 Functional Requirements

Table 3.1: Functional Requirements

ID Requirement Target

FR1 Detect and follow black line on white surface 100% FR2 Adjust speed based on line position (PID) Smooth FR3 Handle straight lines 100% FR4 Handle curves (radius > 15 cm) 100% FR5 Handle 90-degree turns 95%

3.2.2 Performance Requirements

Instruction

Create table with speed, accuracy, response time, battery life targets

8

3.2.3 Safety Requirements (K7)

Instruction

Create table: no exposed voltage, emergency stop, overcurrent protection, no sharp edges, stable operation

3.2.4 Cost Constraints (PO11)

Table 3.2: Budget Breakdown

Category Budget (BDT) Justification

Microcontroller 500 Arduino Nano

Sensors 300 5× IR sensors

Motors & Driver 800 2× DC + L298N

Battery 600 7.4V Li-ion

PCB 400 Local vendor

Chassis 200 Acrylic

Misc. 200 Wires, connectors

Total 3000

3.2.5 Requirements Traceability Matrix

Instruction

Create table mapping each requirement to design element and verification method

Assessment Link:

PO3, PO11 - Comprehensive needs analysis (P2, P6, A2)

3.3 System Architecture and Detailed Design

3.3.1 Overall System Architecture

Instruction

Draw block diagram: Battery → Regulator → MCU → Sensors/Motors. Explain each block.

9

Figure 3.1: System Block Diagram

3.3.2 Sensor Subsystem Design

[1]

Instruction

Include: (1) Selection process table (options, pros/cons), (2) TCRT5000 specifications, (3) Array configuration diagram (5 sensors, 2cm spacing), (4) Position encoding table, (5) Weighted position formula with example, (6) Circuit diagram

Position Encoding Algorithm

Position =��1 × (−2) +��2 × (−1) +��3 × (0) +��4 × (1) +��5 × (2)

��1 + ��2 + ��3 + ��4 + ��5(3.1)

where ���� = 1 if sensor detects black, 0 if white.

3.3.3 Microcontroller Subsystem

Instruction

Include: (1) Selection justification, (2) Pin allocation table, (3) Pin connection diagram

3.3.4 Motor Driver Subsystem

Instruction

Include: (1) L298N selection justification, (2) Specifications, (3) Connection diagram, (4) Motor control truth table

3.3.5 Motor and Wheel Selection

Instruction

Include: (1) Motor selection, (2) Specifications, (3) Speed calculation:

Circumference = �� × �� (3.2)

Speed (m/s) =RPM×Circumference

60(3.3)

10

3.3.6 Power Supply Subsystem

Instruction

Include: (1) Battery selection table, (2) Power budget table (component, voltage, current, power), (3) Runtime calculation, (4) Voltage regulator circuit, (5) Power distribution diagram

3.3.7 PCB Design

Instruction

Include: (1) Why custom PCB, (2) Specifications (size, type, copper thickness), (3) Design process, (4) Layout screenshot with annotations

Assessment Link:

PO3 - Comprehensive design solution (K5)

3.4 PID Control Algorithm Design

3.4.1 Control System Overview

Instruction

Draw control loop: Setpoint → Error → PID → Motors → Sensors (feedback)

3.4.2 Error Calculation

Instruction

Explain with formula and example

3.4.3 Proportional Control

[2]

���������� = ���� ×error (3.4)

3.4.4 Integral Control

[2]

���������� = ���� ×

∑︁

(error× ����) (3.5) 11

3.4.5 Derivative Control [2]

���������� = ���� ×(error−previous error)

����(3.6)

3.4.6 Complete PID and Motor Speed

PID output = ���������� + ���������� + ���������� (3.7)

Left speed = base speed+PID output (3.8)

Right speed = base speed−PID output (3.9)

3.4.7 PID Tuning Strategy

Instruction

Describe step-by-step: (1) Tune Kp only, (2) Add Ki, (3) Add Kd. Include tuning table template

3.4.8 Algorithm Flowchart

Instruction

Draw complete flowchart from initialization to motor control loop

Assessment Link:

PO1 - Applies control theory (K2, K3, K4)

3.5 Design Calculations and Analysis

3.5.1 Motor Torque Requirements

Instruction

Calculate: (1) Force = mass × acceleration, (2) Friction force, (3) Required torque = F × radius, (4) Safety factor check

12

�� = �� ×�� (3.10)

���� �������������� = �� × �� ×�� (3.11)

������������������ = ������������ ×�� (3.12)

3.5.2 Power Consumption Analysis

Instruction

Calculate power for sensors, Arduino, motors. Show runtime calculation:

Runtime (hours) =Battery capacity (mAh)

Average current (mA)(3.13)

3.5.3 Voltage Regulator Heat Dissipation

������������ ���������� = (������ −��������) × �������� (3.14)

3.5.4 Turn Radius Calculation

Instruction

Use differential drive kinematics formula

3.5.5 Cost Analysis

Instruction

Detailed component cost table. Compare with commercial alternatives

Assessment Link:

PO1, PO11 - Mathematical analysis and economic evaluation

3.6 Alternative Designs Considered

Instruction

Describe 2-3 alternatives: (1) Simple on-off with 2 sensors, (2) PID with 3 sensors, (3) Selected design (PID with 5 sensors). For each: advantages, disadvantages, decision. Include decision matrix.

13

Assessment Link:

PO3, PO11 - Evaluates multiple solutions

3.7 Risk Assessment

Table 3.3: Risk Register

ID Risk Likelihood Impact Mitigation

R1 Component unavailable Medium High Identify alternatives early R2 PID tuning takes long High Medium Allocate 2 weeks, document systematically

R3 Battery damage Medium High Use proper charger, monitor voltage

Instruction

Detail mitigation for top 3 risks

Assessment Link:

PO6 - Responsible engineering (K7)

14

Chapter 4

Implementation

4.1 Resource Planning and Management

4.1.1 Component Procurement

Instruction

Include: (1) Procurement timeline table, (2) Challenges and solutions, (3) Budget tracking (planned vs actual)

4.1.2 Schedule and Timeline Management

Instruction

Include Gantt chart, project phases table, critical path, actual vs planned

Assessment Link:

PO11 - Project management (A1)

4.2 Breadboard Prototyping

Instruction

Include: (1) Purpose, (2) Photo of breadboard setup, (3) Testing checklist, (4) Issues found table (issue, symptom, cause, solution), (5) Lessons learned

Assessment Link:

PO3 - Iterative design

15

4.3 PCB Assembly

4.3.1 PCB Fabrication

Instruction

Include: (1) Specs submitted, (2) Process timeline, (3) Quality inspection checklist, (4) Photos of bare PCB

4.3.2 Component Soldering

Instruction

Include: (1) Tools used, (2) Soldering sequence, (3) Quality checks, (4) Challenges, (5) Photos of assembled PCB

4.4 Mechanical Assembly

Instruction

Include: (1) Chassis fabrication, (2) Component mounting, (3) Assembly challenges, (4) Photos

4.5 Software Development

4.5.1 Development Environment

Instruction

Describe IDE, libraries

4.5.2 Code Structure

Instruction

Include: (1) Program flowchart, (2) Function descriptions, (3) Key code snippets

Listing 4.1: PID Calculation Example

/ / C a l c u l a t e e r r o r

f l o a t e r r o r = 0 − p o s i t i o n ; / / S e t p o i n t i s 0 ( c e n t e r )

/ / PID c a l c u l a t i o n s

f l o a t P = Kp ∗ e r r o r ;

16

i n t e g r a l += e r r o r ∗ d t ;

f l o a t I = Ki ∗ i n t e g r a l ;

f l o a t d e r i v a t i v e = ( e r r o r − p r e v e r r o r ) / d t ;

f l o a t D = Kd ∗ d e r i v a t i v e ;

/ / Combined PID o u t p u t

f l o a t P I D o u t p u t = P + I + D;

/ / Motor s p e e d s

i n t l e f t s p e e d = b a s e s p e e d + P I D o u t p u t ;

i n t r i g h t s p e e d = b a s e s p e e d − P I D o u t p u t ;

4.5.3 Debugging Process

Instruction

Include: (1) Tools (serial monitor), (2) Major bugs table, (3) Testing approach

4.6 System Integration

Instruction

Include: (1) Integration sequence, (2) Interface testing, (3) Challenges, (4) Final checklist 17

Chapter 5

Testing and Results

5.1 Testing Methodology

Instruction

Describe: (1) Testing strategy (unit, integration, system), (2) Equipment used, (3) Test environment (track dimensions), (4) Success criteria

5.2 PID Tuning Results

Instruction

Include tuning table:

Table 5.1: PID Tuning Process

Test Kp Ki Kd Observation Action

1 10 0 0 Slow response Increase Kp

2 20 0 0 Better, slight oscillation Increase Kp

3 30 0 0 Too much oscillation Reduce, add Ki 4 20 0.5 0 Good, no steady error Add D

5 20 0.5 10 Smooth! Final values

Instruction

Include photos/videos at different tuning stages

Assessment Link:

PO1 - Applies control theory practically

18

5.3 Functional Testing

Table 5.2: Functional Test Results

ID Test Case Expected Actual Result

FT1 Straight line following Stay on line Stayed on line Pass FT2 Gentle curve (R=20cm) Smooth turn Smooth turn Pass FT3 Sharp curve (R=15cm) Navigate curve Slight deviation Pass FT4 90° turn Turn successfully Success 19/20 tries Pass FT5 Line end detection Stop immediately Stopped Pass

Instruction

Include photos of each test

Assessment Link:

PO3 - Validates functional requirements

5.4 Performance Testing

Instruction

Include: (1) Speed vs PWM table/graph, (2) Accuracy measurements, (3) Response time, (4) Battery life results, (5) Target vs achieved comparison

5.5 Safety and Reliability Testing

Instruction

Include: (1) Safety tests (voltage, current, emergency stop, stability), (2) Reliability tests (continuous operation), (3) Results table

5.6 Preventive Maintenance Procedures

Instruction

Include: (1) Common failure modes, (2) Fault diagnosis flowchart, (3) Troubleshooting table, (4) Maintenance schedule, (5) Spare parts list

19

Table 5.3: Troubleshooting Guide

Symptom Possible Cause Solution

Robot not moving Battery dead Charge battery Erratic movement Loose sensor connection Check connections Goes off track Sensors dirty Clean sensors One motor not working Motor driver issue Check L298N

5.7 Analysis and Discussion

Instruction

Write 2-3 pages: (1) Why results match/differ from expectations, (2) Sources of error, (3) Strengths of solution, (4) Limitations discovered, (5) How P1-P7 complex problem was solved, (6) Compare with Chapter 1 objectives

Assessment Link:

PO1, PO3 - Critical analysis and validation

20

Chapter 6

Project Management and Teamwork

6.1 Project Planning

Instruction

Include: (1) Initial Gantt chart, (2) Milestones, (3) How schedule managed, (4) Changes and why

Assessment Link:

PO11 - Management principles

6.2 Budget Management

Instruction

Include: (1) Budget estimate vs actual, (2) Overruns/savings with reasons, (3) Cost optimization decisions

6.3 Team Organization and Roles

Instruction

Include: (1) Team structure, (2) Responsibilities, (3) Individual contributions, (4) Collaboration with others

Assessment Link:

PO9 - Teamwork documentation

6.4 Team Collaboration and Leadership

Instruction

Include: (1) Communication methods, (2) Meeting frequency, (3) Decision-making, (4) Conflict resolution, (5) Leadership examples

21

6.5 Project Execution and Control

Instruction

Include: (1) Progress tracking, (2) Issues and resolutions, (3) Quality control, (4) Schedule adherence

6.6 Lessons Learned

Instruction

Reflect on: (1) What worked well, (2) What to improve, (3) Skills developed, (4) Advice for future students

22

Chapter 7

Economic and Sustainability Analysis

7.1 Cost Analysis

7.1.1 Development Costs

Instruction

Detailed breakdown by category

7.1.2 Operational Costs

Instruction

Power consumption, maintenance, replacements, annual cost

7.1.3 Life Cycle Cost

Instruction

Expected lifespan, total ownership cost

7.2 Economic Viability

Instruction

Include: (1) Market analysis, (2) Pricing strategy, (3) Break-even analysis, (4) ROI estimation, (5) Comparison with alternatives

7.3 Sustainability Assessment

7.3.1 Environmental Impact Assessment

Instruction

Analyze: (1) Energy efficiency, (2) Materials footprint, (3) End-of-life disposal, (4) Recyclability, (5) Carbon footprint

23

7.3.2 Social Impact

Instruction

Analyze: (1) Benefits to users/society, (2) Safety, (3) Accessibility, (4) Educational value

7.4 Societal and Environmental Impact Checklist

Instruction

Complete both checklists below with Low/Medium/High assessments and justifications

Environmental Impact Checklist

Area Questions Assessment Justification

Energy

Consumption

Is device energy-efficient? Can it use renewable energy?

[L/M/H Impact]

[Explain]

Material Selection Are materials recyclable? Any hazardous materials?

Manufacturing Is process energy-intensive? Waste generated?

[L/M/H Impact]

[L/M/H Impact]

[Explain] [Explain]

Lifespan & Durability

Expected lifespan? Can be repaired?

[L/M/H Impact]

[Explain]

End-of-Life How disposed? E-waste considerations?

Carbon Footprint Estimated emissions? Compared to alternatives?

Societal Impact Checklist

[L/M/H Impact]

[L/M/H Impact]

[Explain] [Explain]

Area Questions Assessment Justification

Safety Risk to users? Standards compliance?

[L/M/H Risk] [Explain]

Accessibility Usable by people with disabilities? Affordable?

24

[L/M/H Access]

[Explain]

Employment Replace jobs? Create opportunities?

[Pos/Neu/Neg] [Explain]

Education Educational value? Teaching tool potential?

[L/M/H Value]

[Explain]

Ethics Privacy concerns? Dual-use potential?

[Issues: Y/N] [Explain]

Cultural Culturally appropriate for Bangladesh?

Overall Impact Summary

Top 3 Positive Impacts:

• Your impact 1

• Your impact 2

• Your impact 3

Top 3 Negative Impacts:

• Your concern 1

• Your concern 2

• Your concern 3

Mitigation Strategies:

• For Impact 1: [Your mitigation]

• For Impact 2: [Your mitigation]

• For Impact 3: [Your mitigation]

[L/M/H Align]

[Explain]

Overall Sustainability Score: [Low / Medium / High]

Instruction

Justify your score in 2-3 sentences

Assessment Link:

PO6, PO7, PO8 - Societal, environmental, ethical responsibility (K7) 25

7.5 Ethical Considerations

Instruction

Discuss: (1) Ethical issues in design, (2) Privacy concerns, (3) Safety responsibilities, (4) IP considerations, (5) Responsible innovation

26

Chapter 8

Conclusions and Recommendations

8.1 Summary of Work

Instruction

Write 1 page brief recap: problem, approach, key features, main results

8.2 Achievement of Objectives

Instruction

Revisit EACH objective from Chapter 1. State how achieved (with evidence) or why not

Assessment Link:

PO3 - Validates solution effectiveness

8.3 Contributions and Significance

Instruction

Describe: (1) Novel aspects (A3, A5), (2) Improvements over existing solutions, (3) Technical contributions, (4) Practical applications (A4)

8.4 Limitations

Instruction

Honestly state: (1) Technical limitations, (2) Assumptions and implications, (3) Constraints that affected outcomes

27

8.5 Future Work and Recommendations

Instruction

Suggest: (1) Possible enhancements (obstacle detection, wireless, path planning), (2) Different approaches, (3) Scalability, (4) Recommendations for future students

8.6 Personal Reflection and Learning

Instruction

Reflect on: (1) Technical skills gained, (2) Management skills, (3) Challenges overcome, (4) What you’d do differently, (5) Career preparation

Assessment Link:

PO12 - Lifelong learning awareness

28

Appendix A

Complete Circuit Schematics

Instruction

Include high-resolution, clearly labeled schematics. One figure per major subsystem. Figure A.1: Complete System Schematic

29

Appendix B

PCB Layout Files

Instruction

Include: top layer, bottom layer, silkscreen, drill file images. Also include Bill of Materials table.

Table B.1: Bill of Materials (BOM)

S/N Component Value Qty Price (BDT)

1 Arduino Nano - 1 450

2 TCRT5000 Sensor - 5 200

3 L298N Driver - 1 280

4 BO Motor 100 RPM 2 240

Continue... .. .. .. ..

Total: 3000

30

Appendix C

Complete Source Code

Instruction

Include well-commented, complete Arduino code. Organize by functions.

Listing C.1: Complete Line Follower Code

/ / ============ LIBRARIES ============

/ / [ Add y o u r i n c l u d e s ]

/ / ============ PIN DEFINITIONS ============ # d e f i n e SENSOR1 A0

# d e f i n e SENSOR2 A1

/ / [ C o nti n u e . . . ]

/ / ============ GLOBAL VARIABLES ============ f l o a t Kp = 2 0 . 0 ;

f l o a t Ki = 0 . 5 ;

f l o a t Kd = 1 0 . 0 ;

/ / [ C o nti n u e . . . ]

/ / ============ SETUP ============

void s e t u p ( ) {

/ / [ Your i n i t i a l i z a t i o n c o de ]

}

/ / ============ MAIN LOOP ============

void l oop ( ) {

/ / [ Your main c o de ]

}

/ / ============ FUNCTIONS ============

/ / [ Your f u n c t i o n s ]

31

Appendix D

Component Datasheets

Instruction

Include key datasheets (first 2-3 pages) for: TCRT5000, L298N, Arduino Nano, motor 32

Appendix E

Test Data

Instruction

Include raw data tables, additional graphs not in main report 33

Appendix F

Project Photographs

Instruction

Comprehensive photo documentation: breadboard, PCB fabrication stages, assembly, final robot from multiple angles, testing

34

Appendix G

Budget Details

Instruction

Itemized breakdown with scanned receipts or quotes 35

Appendix H

Project Timeline

Instruction

Include detailed Gantt chart

36

Appendix I

Risk Register

Instruction

Complete risk assessment matrix with all identified risks 37

Appendix J

Meeting Minutes

Instruction

Key meeting notes with dates, attendees, decisions made 38

Appendix K

Video Demonstration Links

Instruction

Provide links to: (1) Final demo video, (2) PID tuning attempts, (3) Testing videos. Include QR codes if possible.

39

Assessment Evidence Summary

PO1: Knowledge Application

Assessed Through:

• Chapter 2: Applied knowledge of electronics, microcontrollers, sensors, control theory (K1-K4)

• Chapter 3.4-3.5: Mathematical analysis, PID theory, engineering calculations (K2, K3) • Chapter 1.2, 5.7: Complex problem identification and solution (P1-P7) Evidence: Theoretical foundation, design calculations, results analysis

PO3: Design Solutions

Assessed Through:

• Chapter 3.3: Complete system design addressing functional, safety, usability requirements (K5)

• Chapter 3.6: Alternative designs evaluated, informed selection

• Chapter 5: Functional solution validated through testing

• Chapter 7.3: Societal and environmental considerations addressed (K7)

Evidence: Comprehensive design documentation, testing validation, sustainability assessment

PO11: Management & Economics

Assessed Through:

• Chapter 4.1: Resource planning and budget management (A1)

• Chapter 6: Complete project management (planning, execution, control, teamwork) • Chapter 7.1-7.2: Economic analysis and cost-benefit evaluation

• Chapter 4.1.2: Schedule management, timeline adherence

Evidence: Gantt charts, budget tracking, cost analysis, meeting documentation 40

Complex Engineering Characteristics

• Knowledge Profile (K1-K8): Chapters 2, 3 demonstrate all eight areas • Complex Problems (P1-P7): Chapter 1.2 identifies, entire project addresses

• Complex Activities (A1-A5): Chapters 4, 6 document diverse resources, innovation, novel applications

41

References

[1] IEEE Computer Society, “Guide to face recognition systems,” https://www.computer.org/ face-recognition-guide, 2023, accessed: Jan. 5, 2026.

[2] N. S. Nise, Control Systems Engineering, 7th ed. Hoboken, NJ: John Wiley & Sons, 2015. 42